Nordkappterminalen, Honningsvåg

Finnmark, Norway

PORT/TERMINAL INFORMATION BOOKLET

NOVEMBER 2019

PREFACE

Purpose

The  purpose  of  this  booklet  is  to  assist  oil  tankers  delivering  oil  products  to Honningsvåg Terminal, North cape.

Unless otherwise stated, the limitations and recommendations in the booklet are intended for oil tankers only.

Introduction

This booklet contains general information and safety regulations for vessels delivering oil products to the terminal.

The ship is responsible for safe operation on board while moored to the terminal. In case of an incident outside the ship, we seek your full co-operation and understanding of the safety requirements set out in the Ship/Shore checklist and in these regulations.

Failure to comply with these regulations will lead to interruption of the discharging/loading operation and/or removal of ship from berth. The discharging/loading operation may be restarted, but this requires a written assurance from the master that control has been established. The vessel is responsible for any cost and delays caused by non-compliance of the safety procedures.

It is the Master’s responsibility to ensure that all personnel on the ship follow the safety regulations.

The content of this booklet is in addition to, and does not supersede or replace, any information contained in the national and/or local port regulations.

The information contained herein is believed to be correct at the time of issue. However, no responsibility is accepted by Bunkeroil AS, its principals or agents for the accuracy of anything contained herein, or contained within any supplemental publication.

1   TABLE OF CONTENTS

Innhold

PREFACE	2

Purpose	2

Introduction	2

1   TABLE OF CONTENTS	3

2   TERMINAL INFORMATION AND SERVICES	3

2.1  General	3

2.2  Berth information	4

2.3  Pilots	4

2.4  Pre - loading/ discharging conference	4

2.5  Inspections	5

2.6  Ship/ shore safe access	5

2.7  Access to the terminal	5

2.8  Use of alcohol, drugs and strong prescribed medicine	5

2.9  Concern for neighbouring residential areas	5

2.10 General cargo/ supplies	5

2.11 Bunkering	6

2.12 Fresh water	6

2.13 Garbage and slop/sludge	6

3   COMMUNICATION	6

3.1  Communication equipment	6

3.2  Communication procedures	7

3.3  Useful contact details	7

4   PRE-ARRIVAL EXCHANGE OF INFORMATION	8

5 LIMITING CONDITIONS FOR BERTHING/ UN-BERTHING OF TANKERS	9

5.2  Current restrictions	9

5.3  Visibility restrictions	9

5.4  Swell restrictions	9

5.5  Electrical storms/ lightning	9

5.6  Berth approach	10

6   BERTHING/ UN-BERTHING	10

6.1  Tug	10

6.2  Mooring plans	10

7   GENERAL PRECAUTIONS	10

7.1  Removal of blind flanges	10

7.2  Accidental oil spillage and leakage	10

7.3  Fire fighting equipment	11

7.4  Scupper plugs	11

7.5  Spill containment	12

7.6  Portable electronic equipment	12

7.7  Smoking and the use of open fire	12

7.8  Emergency towing/ fire wires	12

7.9 Ullaging and sampling	12

7.10 Overflow protection	12

7.11 Emergency shutdown	13

7.12 Pressure surge precautions	13

7.13 Ballast discharge	13

7.14 Ballasting	13

7.15 De-gassing and/ or tank cleaning	14

7.16 Repairs	14

7.18 Hazardous/hot work	14

7.19 Moorings and deck watch	14

7.20 Draining of loading hoses/ arms	14

8   ALARM INSTRUCTIONS AND ACTIONS TO BE TAKEN	15

8.1  Fire on board your vessel while berthed	15

8.2  Fire at the terminal	15

8.3  Action onboard ship	15

8.4  Action onshore	15

9   Appendixes	16

9.1 APPENDIX 01 - MOORING PLAN, LOA 50-80 m	17

9.2 APPENDIX 02 -MOORING PLANS, LOA 100 m – 120 m	18

9.3 APPENDIX 03 MOORING PLANS, LOA 120 m – 170 m	20

9.4 	APPENDIX 04 - LOCATION OF MANIFOLDS and GENERAL LAYOUT	21

9.5 	APPENDIX 05 -Legend and notes	22

9.6     APPENDIX 06 - EMERGENCY EVACUATION	23

2   TERMINAL INFORMATION AND SERVICES

2.1  General

The geographical position of the terminal is N 70 59 E 25 56

Distance to residential areas is approx. 1100 metres.

The terminal is ISPS approved.

2.2  Berth information

General limitations:

- Maximum vessel size:  Approx. 75 000 tdw

- LOA: Approx 225 m -draft limitation only

- Max draft: 9.5 m. UKC (Under Keel Clearance) 0.5 m.

Extreme weather conditions can occur.

Berth data:

- Depth at lowest astronomical tide, LAT 10.0m. (berth front).

- Length of berth front: 48 m.

- Fenders at berth corner: No

- Fender capacity: Dumper tires. Capacity not established.

2.3  Pilots

Pilotage is compulsory for all vessels above 500 tons displacement or if under the command of a foreign captain/ officer that does not hold a waterway certificate (farledsbevis) for the local waters.

Vessels route to and from the terminal must comply with all regulations regarding the use of pilot within Norwegian territorial waters. A copy of these regulations can be obtained from the pilots on request.

2.4  Pre - loading/ discharging conference

A pre-loading / discharging conference shall be held between the Bunkeroil AS

representative and the ship’s cargo officer. In order to aid the overall safe management of the operation, an inspection based on the “Ship / Shore safety Checklist shall be carried out before any loading or discharging. Repetitive checks will be undertaken at intervals agreed on during the pre-transfer conference. Material safety data sheets (MSDS) are available from the terminal office on request.

2.5  Inspections

Norway is a signatory of the Memorandum for Port State Control, and in addition to terminal inspections, masters can expect governmental inspections to be undertaken, aimed at confirming that the ship meets all relevant international standards.

2.6  Ship/ shore safe access

The vessel has to provide its own gangway, and safety net is mandatory.

2.7  Access to the terminal

Only persons authorized by the terminal/Port Authority of North cape are allowed to enter the ISPS terminal area. Unauthorized persons are prohibited from entering the berth or boarding the vessel. Any unauthorised person will be denied access to the terminal.

2.8  Use of alcohol, drugs and strong prescribed medicine

The use of alcohol, drugs and/or strong prescribed medicine is prohibited at the berth. Any person that appears to be under the influence of alcohol, drugs or strong prescribed medicine will be prohibited from entering the terminal facilities. No one under the influence of alcohol, drugs or strong prescribed medicine, are allowed to participate in any operational activities at the terminal.

2.9  Concern for neighbouring residential areas

When a residential area is situated close to the terminal, every effort aimed at reducing a negative environmental impact on our neighbours is appreciated. During your vessel’s stay at the terminal we recommend that you consider minimising the use of ventilators/ fans without jeopardizing the safety on board. Noise and soot from the vessel’s funnel should be minimised as much as practically possible.

2.10 General cargo/ supplies

Supplies or ship’s provisions may be transported on to the terminal berth if permitted by the terminal manager and if it is not violating the terminal safety regulations.

Weight restrictions are as follows: The berth can carry the load of a truck. Capacity: 10 kN/m2, point load 15 kN/m2.

Ship’s crane may be used for lifting ship’s supplies and equipment. However, no lifting of any equipment that may ignite sparks, such as drums, steel, pipes etc., may take place during cargo operations.

2.11 Bunkering

No bunker barges are allowed alongside vessels moored alongside the berth.

The import pipes are DN 200 (8”) for HFO, and DN 150 (6”) for MGO and ADO

Bunker oil (MGO, HFO,ADO) to vessels are supplied through 2”-3” hoses, at a rate from Qmin-200L/min equivalent 11,5 m3/h (14 tons/h) to Qmax- 3800L/min equivalent 230 m3/h (270 ton/h)

All export pipes are connected with DN 100 Coriolis flowmeter.

Lubricants Oil.

Mobil lubricants.       https://www.bunkeroil.no/

2.12 Fresh water

Fresh water is supplied at the berth by the terminal.

2.13 Garbage and slop/sludge

No garbage or other materials, neither liquid nor solid, shall be discharged overboard from a vessel alongside the terminal. Common organic waste can be deposited in a container on the berth, but arrangements should be made with the ship’s agent for an appointed, certified company to collect other types of waste, slop or sludge while the vessel is alongside the berth.

Disposal of dirty ballast water and/ or slops into the port basin, as well as any pollution of Norwegian coastal waters, will result in heavy fines.

3   COMMUNICATION

3.1  Communication equipment

Telephone, portable VHF/UHF and radiotelephone systems should comply with the appropriate safety requirements.

When VHF/UHF or radiotelephone systems are used, units should preferably be portable and carried by the responsible officer on duty and the responsible person ashore, or by persons who can contact their respective superiors immediately.

Communication between the responsible officer and the terminal representative should be continuously manned by persons who immediately can contact their superiors. Necessary information concerning the selected system of communication and/ or telephone numbers to be used, should be stated in the appropriate form.

This form should be signed by both ship and shore representatives.

3.2  Communication procedures

To ensure the safe control of operations at all times, it should be the responsibility of both parties to establish, agree on and maintain a reliable communication system.

For terminal contact details, useful telephone numbers and VHF channels, see figure below.

3.3  Useful contact details

|  | Useful contact information |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Phone | Cell phone | Fax | VHF channel | E-mail |
| Emergency |  |  |  |  |  |
| Fire brigade | 110 |  |  |  |  |
| Police | 112 |  |  |  |  |
| Ambulance | 113 |  |  |  |  |
| Emergency channel |  |  |  | 16 |  |
| Terminal operator |  |  |  |  |  |
|  |  | 950 22167 |  |  | havna@nordkapp.kommune.no |
|  |  | 950 22167 |  |  | havna@nordkapp.kommune.no |
| Bunker Oil representative Honningsvåg | Nordkapp Havn: (+47) 784 76 450 | Nordkapp Havn: 950 22167 |  |  | havna@nordkapp.kommune.no |
| Port Authority |  |  |  |  |  |
| Northcape Port Authority | (+47) 784 76 450 | 950 22167 |  | 12,16 | havna@nordkapp.kommune.no |
| Pilot |  |  |  |  |  |
| Lødingen pilot | (+47) 76 98 68 10 |  | (+47) 76 98 68 20 | 13,16 | pilot.lodingen@kystverket.no |
| Northcape/ Honningsvåg Port Authority |  | 0047 784 76450 |  | 12 | havna@nordkapp.kommune.no |
| Ship agent |  |  |  |  |  |
| Nordkapp Godsterminal |  | 0047 784 72 061 |  |  | agent@nordkappgodsterminal.no |
| Quay owner | Tromsø Kullkran AS |  |  |  |  |

4   PRE-ARRIVAL EXCHANGE OF INFORMATION

4.1 Tanker to terminal

The terminal and the port authority should be given ETA by the vessel as soon as possible, and not later than 24 hours prior to arrival. If the sea leg sailing from the last port of call is less than 24 hours, ETA should be given immediately upon departure from the last port of call.

• Number, type and size of hose connections and manifolds

5 LIMITING CONDITIONS FOR BERTHING/ UN-BERTHING OF TANKERS

5.1  Wind restrictions

None stated by the terminal.

In Case of extreme weather conditions occur while the vessel is alongside the terminal, the officers in charge on the terminal and on board the vessel will decide on necessary steps to be taken, i.e. cease loading/discharging operations, drain/disconnect hoses, extra moorings, etc.

5.2  Current restrictions

None stated by the terminal.

5.3  Visibility restrictions

None stated by the terminal.

5.4  Swell restrictions

None stated by the terminal.

5.5  Electrical storms/ lightning

Loading/discharging operations will be suspended on the approach of electrical storms/ lightning, regardless of whether or not an IG (inert gas) and/or vapour control system is in use. All tank openings, tank-venting systems (including IG mast riser isolating valve) and manifold valves must be closed.

5.6  Berth approach

Within the port limits, the vessel should proceed at slow speed. During the vessel’s final approach, the speed towards the berth should be minimized in order to reduce the impact on the fenders.

6   BERTHING/ UN-BERTHING

6.1  Tug

No tug requirements by the terminal or the Port Authority.

6.2  Mooring plans

Vessels is recommended to comply with the enclosed mooring plans, in accordance with vessel’s size and prevailing weather conditions. See appendixes 2 - 4

7   GENERAL PRECAUTIONS

7.1  Removal of blind flanges

- Each tanker- and terminal manifold flange should have a removable blind flange, made of steel, or other approved material such as phenol resin, and preferably fitted with handles.

- Precautions should be taken in order to ensure that - prior to the removal of line blinds from tanker and terminal pipelines - the section between the last valve and the blind flange does not contain cargo under pressure.

- Precautions is to be taken in order to prevent any spillage.

- Blind flanges shall be capable of withstanding the working pressure of the line /system to which they are connected.

- Blind flanges should normally be of a thickness equal to that of the end flanges to which they are fitted.

7.2  Accidental oil spillage and leakage

No oil or mixture containing oil shall be discharged or allowed to escape from a vessel while alongside the terminal. The engine room bilge overboard valve should be closed and locked shut while the vessel is in port. It is important that the surface of the water around the vessel is monitored as a check against any accidental escape of oil.

In order to prevent pollution of coastal waters, and in an endeavour to avoid subsequent heavy claims, the shore installation will - in case of any oil spill from a vessel- take such steps as may be deemed necessary to fight the pollution before it spreads.

If the oil spill is caused by faulty equipment or material on board the vessel, or by negligence on the part of the ship’s personnel, the shore installation shall be indemnified by the ship for any expenses incurred in connection with the preventive action taken.

ANY OIL SPILL MUST BE REPORTED IMMEDIATELY

7.3  Fire fighting equipment

When a tanker is alongside the berth, firefighting equipment is to be ready for immediate use.

Onboard the ship, this is normally achieved by having fire hoses with spray/jet nozzles connected and run out forward and aft of, and adjacent to, the manifold in use. Additional protection against small flash fires should be provided by having a portable dry chemical powder extinguisher available near the manifold.

Considerations should be given to having portable extinguishers available for use adjacent to the berth manifold area.

7.4  Scupper plugs

Before cargo handling commences, all deck scuppers and, where applicable, open drains on the berth, must be effectively plugged so as to prevent spilled cargo escaping into the sea around the tanker or the terminal. Accumulation of water should be drained periodically and scupper plugs replaced immediately after the water has been run off.

7.5  Spill containment

A permanently fitted spill tank, provided with suitable means of draining, should be fitted under all ship-and-shore manifold connections. Should no permanent means be provided, drip trays should be placed under each connection to retain any leakage.

7.6  Portable electronic equipment

Use of cell phones, personal computers, pagers and cameras have no restrictions.

7.7  Smoking and the use of open fire

Smoking and the use of open fire is strictly prohibited on the berths, on the shore terminal area and on board vessels alongside the berths. Exemption is made for those spaces on board ships, designated as “Smoking Area” by the master, and agreed on by the terminal representative.

7.8  Emergency towing/ fire wires

Fire wires are to be positioned at the offshore bow and quarter whilst alongside, to enable tugs to connect readily in an emergency situation. The towing eyes must be lowered to about 1 metre above water level and sufficient slack kept on deck.

7.9 Ullaging and sampling

Quality control sampling is always performed prior to discharging. Wherever possible, ullaging, dipping and sampling of ship’s tanks should be done by using closed sampling equipment. Under no circumstances are shore personnel/surveyors to open any tank without approval from the ship’s duty officer and a representative of the ship’s staff being present. Cargo tanks should always be depressurised, using the fitted tank venting system.

7.10 Overflow protection

In order to prevent spillage from overflow of cargo tanks during loading and discharging operations, it is imperative that the high-level alarms are in working order.

Each individual high-level alarm in the cargo tanks should be tested and the alarm settings on the cargo monitoring equipment should be checked.

The discharge plan should include instructions for ensuring that the volume level in all cargo tanks are reduced, prior to commencement of discharge operations, to an ullage which ensures high-level alarm activation in case of runback into any tank.

7.11 Emergency shutdown

Transfer operations shall be halted immediately in the event of:

- Cargo spillage or suspected cargo spillage.

- Fire or explosion on the vessel or on the terminal.

- In case of power outage at the terminal.

In case of malfunction of the ship / shore communication system, if the mooring lines repeatedly are not properly tended, if the deck watch is absent, or if any other un-normal situation occurs, transfer operations may be discontinued if the failure is not corrected immediately.

7.12 Pressure surge precautions

In the event of an emergency shutdown, the import valves for the terminal tanks will close when the emergency switch has been activated. Alarms are installed.

The pipeline systems are designed to give acceptable surge conditions. Systems on board the vessel must be able to cope with such conditions.

7.13 Ballast discharge

The port authority does not permit ballast discharging when alongside the berth..

7.14 Ballasting

The master is responsible for ensuring that the vessel at all times is ready for manoeuvring, with respect to draft, trim, stability and propeller immersion.

7.15 De-gassing and/ or tank cleaning

No de-gassing or tank cleaning is allowed while discharging petrol/gasoline. When loading/discharging other cargo qualities, permission must be obtained from the terminal manager.

7.16 Repairs

When the ship is alongside, only minor repairs should be carried out, and the ship’s engines shall not be immobilised. At any time, the port’s safety regulations are to be followed. In exceptional circumstances immobilisation may be permitted, subject to approval of the Port Authorities and the terminal manager

7.18 Hazardous/hot work

No hazardous/hot work, creating sparks or fire, are permitted while the vessel is alongside, unless permitted by the terminal manager.

7.19 Moorings and deck watch

A qualified member of the vessel’s staff must at all times monitor the vessel’s loading/discharging and her moorings, which must be kept taut at all times.

7.20 Draining of loading hoses/ arms

The ship must be able to drain loading hoses/ arms into a cargo tank when completing cargo operations. The terminal will strip the shore system when discharging is completed.

8   ALARM INSTRUCTIONS AND ACTIONS TO BE TAKEN

8.1  Fire on board your vessel while berthed

Should fire be detected on board a vessel while berthed, the vessel should sound one or more blasts of the ship’s siren, supplemented by a continuous sounding of the ship’s general alarm system.

Notify the terminal immediately.

8.2  Fire at the terminal

The terminal fire alarm will be sounded continuously. The local fire department will automaticly be alarmed and will, in the event of an emergency, be in charge/ coordinate emergency response.

8.3  Action onboard ship

In case of fire on board your ship:

- Raise the alarm

- Inform the terminal

- Fight fire and prevent fire spreading

- Cease all cargo operations, drain hoses and then close all valves

- Stand by to disconnect hoses

- Bring engines to standby and be ready for departure

In case of fire on other ships or on the terminal:

- Cease all cargo operations, drain hoses and then close all valves

- Stand by to disconnect hoses

- Bring engines to standby and be ready for departure

8.4  Action onshore

In case of fire on the terminal:

- Sound the alarm

- Cease all cargo operations, drain hoses and then close all valves

- Inform all ships alongside the terminal

- Fight fire and prevent the fire from spreading

NB! In case of fire on board your vessel, other vessels or the terminal, the terminal fire alarm will be activated immediately.

9   Appendixes

9.1   	APPENDIX 01 - MOORING PLAN, LOA 50-80 m

9.2   	APPENDIX 02 - MOORING PLANS, LOA 100 m – 120 m

9.3   	APPENDIX 03 - MOORING PLANS, LOA 120-170 m

9.4 	APPENDIX 04 - LOCATION OF MANIFOLDS and GENERAL LAYOUT

9.5    APPENDIX 05 – LEGEND AND NOTES

9.6	APPENDIX 05 - EMERGENCY EVACUATION

9.1 APPENDIX 01 - MOORING PLAN, LOA 50-80 m

9.2 APPENDIX 02 -MOORING PLANS, LOA 100 m – 120 m

9.3 APPENDIX 03 MOORING PLANS, LOA 120 m – 170 m

9.4 	APPENDIX 04 - LOCATION OF MANIFOLDS and GENERAL LAYOUT

9.5 	APPENDIX 05 -Legend and notes

9.6     APPENDIX 06 - EMERGENCY EVACUATION